הסבר של החלוניות של מה שקלטנו מה-

wireshark shark בצאט

Protocol Analysis of a Multi-User TCP Chat System: Application, Transport, and Network Layer Breakdown:

1. Application Layer

The capture reveals a custom Chat Server protocol operating on port 10000. The interaction follows a specific command-and-response flow:

- Initialization: Upon connection, the server sends a "welcome to the chat" message and prompts the user to "enter name:".

- User Identity: Four users are identified through their input: Ofir (Port 58197) , Roony (Port 52837) , Hodaya (Port 51542) , and Emily (Port 51546).

- Menu System: The server provides a menu with three options: 1) to connect, 2) leave, and 3) exit.

- Session Management:

  - Chat Requests: Users initiate chats by typing another username (e.g., Ofir requests Roony).

  - Acceptance: The recipient must type "accept" to begin the session.

  - Notifications: The server broadcasts status updates, such as "Emily joined the chat" or "Ofir left the chat".

- Message Relay: The server acts as a middleman, prepending the sender's name to the message (e.g., "Roony: hello").

2. Transport Layer (TCP)

The communication relies on the Transmission Control Protocol (TCP) to ensure reliable data delivery.

- Handshake: Each client performs a standard three-way handshake ([SYN], [SYN, ACK], [ACK]) to establish a connection with the server at 127.0.0.1:10000.

- Data Pushing: Most data packets use the [PSH, ACK] flags, which instructs the TCP stack to pass the data to the application immediately rather than waiting for the buffer to fill.

- Sequencing and Acknowledgement:

  - The server tracks data using Sequence (Seq) and Acknowledgment (Ack) numbers.

  - For example, after the server sends 20 bytes of "welcome" data , the client acknowledges receipt by setting its Ack number to 21.

- Port Mapping:

  - Server Port: 10000.

  - Client Ports: 58197, 52837, 51542, and 51546.

3. Network Layer (IPv4)

The network layer handles the addressing and routing of packets.

- Protocol: Internet Protocol version 4 (IPv4).

- Addressing: All communication occurs on the Loopback address (127.0.0.1). This indicates that the client and server programs are running on the same physical machine.

- Traffic Flow:

  - Source: 127.0.0.1.

  - Destination: 127.0.0.1.

- Fragmentation: There is no evidence of IP fragmentation, as the data segments (ranging from 4 to 48 bytes) are well below the standard MTU size.

Would you like me to create a summary table comparing the specific timestamps and data lengths for each of the four users?